APPROVED ASSEMBLY OF SOLID HARDWOOD FLOOR WITH PLYWOOD

• Make sure subfloor is clean, dry, and flat. If some areas require fill

patching or leveling use a self leveling underlayment.

• Must check subfloor for excessive moisture. If a moisture barrier is

needed use a suitable vapor barrier or moisture sealer.

• Loose lay Acoustical Decoupler on subfloor, do not glue.

• Make sure Acoustical Decoupler covers the perimeter of the

entire subfloor and around any protrusions.

• Always install black side down.

• The Acoustical Decoupler seams must be placed tightly

together and taped with duct tape.

• Loose lay the first layer of plywood (3/4”) on Acoustical

Membrane.

• Install second layer of plywood (1/2”) on top of the first layer of

plywood. The top layer of plywood must be perpendicular to the

bottom layer, joints gapped and offset, top sheet glued (full coverage,

non water based adhesive) to bottom sheet. The top layer fastened with

1” #8 flat head wood screws every 12” with no penetration into the

Acoustical Membrane.

• Nail down hardwood floor on the plywood without penetrating the

Acoustical Membrane.

THIS INSTALLATION WILL CREATE A FLOATING HARDWOOD

FLOOR ON PLYWOOD ASSEMBLY OVER ACOUSTICAL

MEMBRANE